PROPERTY DESCRIPTION

Being all that tract or parcel of land lying and being in Land Lot 361, 18th District, DeKalb County, Georgia and being more particularly described as follows:

Beginning for the same at a ½ inch rebar found on the Easterly Right-of-Way Line of Vermack Road (an 80 feet wide right of way), said point being at State Plane Coordinate (Georgia West Zone) of North: 1,432,664.332; East: 2,251,573.065; thence, leaving said Point of Beginning and the said line of Vermack Road and running,

- South 84° 54' 19" East, 174.52 feet to a ½ inch rebar found; thence,

- North 75° 18' 21" East, 151.29 feet to a ½ inch rebar found; thence,

- North 89° 16' 05" East, 234.50 feet to a ½ inch rebar found; thence, running adjacent to the lines of a subdivision entitled, “Village Mill, Unit Two”, as described on a plat recorded among the Land Records of DeKalb County, Georgia in Plat Book 54, Page 69,

- South 00° 01' 40" East, 532.52 feet to a 1 ½ inch open top pipe found; thence,

- South 88° 52' 51" East, 838.71 feet to a 1 inch open top pipe found; thence,

- South 00° 22' 40" West, 49.18 feet to a 1 inch open top pipe found; thence, running adjacent to the lines of a subdivision entitled, “Heritage at Dunwoody”, as shown on a Final Plat recorded among the aforesaid Land Records in Plat Book 266, Page 1,

- North 88° 47' 51" West, 1,060.20 feet; thence, running adjacent to the property now or formerly owned by Deborah Ann & Scott R. Arant, as described in a deed recorded among the aforesaid Land Records in Deed Book 26563, Page 630,

- North 03° 14' 04" East, 249.82 feet to a ½ inch rebar found; thence,

- North 86° 52' 50" West, 199.82 feet; thence,

- North 65° 32' 05" West, 178.36 feet to a ½ inch rebar found on the aforesaid line of Vermack Road; thence, running with the said line of Vermack Road,

- 216.53 feet along the arc of a curve deflecting to the left, having a radius of 1,343.55 feet and a chord bearing and distance of North 03° 52' 14" East, 216.29 feet to the Point of Beginning, containing 259,751 square feet or 5.9631 acres of land, more or less.

Property is subject to all easements and rights of way recorded and unrecorded.